| CISC | RISC |
| --- | --- |
| Complex instruction set computing | Reduced instruction set computing |
| More number of instructions | Less number of instructions comparitively |
| Complex instructions are made available by the cpu designer | It is programmers responsibility to design complex operations through software |
| Decoder unit size is large | Decoder unit size is small comparitively |
| Power consumption is more | Power consumption is less |
| Internal registers are less | Internal registers are more |
| Memory intensive processing, Frequent access of memory is slow access | Register intensive processing, Frequent access of registers is fast access |
| Instruction sizes and machine cycles required for execution are varied. | Mostly single cycled instructions with fixed length |
| More number of addressing modes | Less number of addressing modes comparitively |

CPU Design Architectures:

- Based on Instruction Set:

- CISC

- RISC

- Based on Memory Organisation:

- Harvard

- Von Neuman

ex:8086,8051…	ex:ARM,AVR…….

ROM

RAM

Von-Neuman:

CPU

- Common pair of address bus and data bus are available to access RAM and ROM

Ex: 8086,ARM7……

Harvard:

ROM

RAM

CPU

- separate pair of address bus and data bus are available to access the ROM and RAM.

- maximize the throughput.

- Ex: 8051,ARM9…..

8051  ARCHITECTURE

Features:

- Employs CISC, Harvard architecture

- 8 bit CPU

- 40 pin DIP(40(physical)=32_I/O+8,64(logical)=(24*2)+16)

- 4KB on chip ROM

- 128 bytes of on chip RAM

- 128 bytes address space reserved for SFRs, available ONLY 21 bytes/sfr’s

- 4 ,8-bit parallel port peripherals

- Two 16 bit timers/counters peripheral

- Full Duplex UART serial peripheral

- On-chip clock oscillator requires ext crystal

- Interrupt Control logic which supports 6 sources,5 vectored interrupts

- Can support up to max 64KB of ROM

- Can support up to addition max 64KB ex-RAM

Figure:  8051 pin diagram

Alternate functionalities of Port3 are as follows:

RXD    Serial data receive pin

TXD    Serial data transmit pin

INT0 External interrupt0 pin

INT1   External interrupt1 pin

T0       Timer0 input pin(used in counter functionality)

T1       Timer1 input pin(used in counter functionality)

WR     write signal to the external RAM

RD      Read signal to the external RAM

- EA  pin:External Access pin

- If EA  pin is connected to Vcc then 8051 CPU fetches the opcodes from On-chip ROM.

- If EA  pin is connected to GND then 8051 CPU fetches the opcodes from External ROM.

Note:1) In 8051 EA  pin need to be connected to Vcc.

Note:2) 8031 is a ROMless version of microcontroller ;In 8031 EA pin need to be connected to GND

- ALE Pin:Address latch enable

- To de-multiplex the address bus and data bus in Port0.

- PSENpin: Program store enable pin

- It is a read control signal to the external ROM.

Power on reset circuit with  switch for run time reset

- Reset in 8051 is active-high reset.

- To reset 8051 ,supply logic 1(Vcc) for minimum two machine cycles.

CRYSTAL CKT

Figure: 8051 Block diagram