- Register addressing mode:

- MOV  A, Rn  ; 8bit in Rn register is copied to Accumulator

- MOV  Rn, A  ; 8bit in Accumulator is copied to Rn register

Example:

- MOV  R0, #56H  ;R0 =56H

MOV   A, R0        ;A   =56h

MOV   R3,A         ;R3 =56H

- Direct addressing mode:

Upper 128 bytes(sfr’s) (only direct addressing )

Lower 128 bytes(RAM) (both directly and indirectly)

Ex:1) MOV 30H,#54H  ; 54H is copied to Internal Ram address 30h

MOV A, 30H      ; the 8 bit present in internal ram 30h is copied to Accumulator

MOV 40H, A     ; The 8 bit present in Accumulator is copied to internal ram address 40h