Immediate addressing mode:

- MOV  A,#data(8bit)

example:

MOV A,#43h ;  43h is copied to accumulator

MOV A,#10   ;   10  is copied to accumulator

MOV A,#’B’   ;  66 or 42h is copied to accumulator

66/16   q=4     r=2

- 8051 can support upto 256 instruction opcodes, but only 255 instruction opcodes are defined.

opcodes      MNEMONIC

00h            NOP

01h            AJMP

02h            LJMP

03h              RR

04h

-

-

74H          MOVA,#data

-

78h          MOV   R0,#data

79H          MOV R1,#data

7ah          MOV R2,#data

--

ffh

- How to find the size of instructions? MNEMONIC OPERAND1, OPERAND2 1byte 1byte 1byte - If any of operand is having the listed registers(A,R0,R1,R2,R3,R4,R5,R6,R7,DPTR) then byte for the operand is along with MNEMONIC.

ROM(program memory)

74h

43h

example:1)

MOV  A, #43H	                                      MOV   A, #43H

74H       43H

1byte        1byte

example:2)

MOV   R0,#23H                            MOV  R0, #23H

1 byte            1byte	                         78h               23h

example:3)

MOV  DPTR, #1234H

1 byte                  2bytes