- Accumulator(A):

- It is an 8 bit register.

- sfr address is E0h.

- default value (or) reset value is 0.

- Accumulator is accessed both bit and byte-wise.

- This register is used in most of the instructions like

data transfer ,rotate,exchange,arithmetic,logical etc..

- Arithmetic register(B):

- 8 bit register.

- sfr address is F0h

- default value is 0.

- B register is accessed both bit and byte-wise.

- This register is mandatory in MUL and DIV instructions.

| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

00h
 7           6           5           4           3           2          1          0

- Data pointer(DPTR):

DPTR is 16 bit register divided into two 8 bit registers.

DPH(8bit)	DPL(8bit)

| 0000 0000 | 0 0 0 0 0 0 0 0 |
| --- | --- |

83h	82h

- DPTR is used to address external RAM or CODE(ROM) memory.

- Can be accessed only byte-wise.

- Interrupt Enable(IE):

- 8 bit register.

- sfr  address is A8h.

- default value is 00h.

- both bit and byte-wise accessable.

- used to enable or disable the interrupts.

- Interrupt priority(IP):

- 8bit register.

- sfr address is B8h.

- default value is 00h.

- both bit and byte-wise accessable.

- used to assign the forced priorities to the interrupts.

1 1 1 1 1 1 1 1 (FFh)

P0(80h)

FFH

P1(90h)

FFH

P2(A0h)

FFH

P3(B0h)

- P0,P1,P2,P3 are accessed both bit and byte-wise.

- Power control register(PCON):

- 8 bit register.

- sfr address is 87h.

- default value is 00h.

- only byte-wise accessable.

- 1) Used to double the baud rate in UART.

2) Used to select power down and idle mode.

Note: 1KB 2^10 Ex1) If the memory is 4KB then width of address bus is 12 bit 4 * KB 2^2 2^10 2^12 Ex2) If the memory is 16KB then width of address bus is 14 bit 16 KB 2^4 2^10 2^14 Ex3)If the address bus is 16bit the maximum memory can be interfaced is 64KB Ans) 2^16 2^6 2^10 64 * 1KB => 64KB

- Program status word(PSW):

- 8bit register.

- sfr address is D0h.

- Default value is 00h.

- both bit and byte-wise accessable.

- 1) Used for selecting the different bank registers.

2) Status of the cpu flags are read by this register.

- Serial port control register(SCON):

- 8bit register .

- sfr address is 98h.

- default value is 00h.

- both bit and byte-wise accessable.

- Used to control and read the status of UART peripheral.

- Serial port Data buffer(SBUF):

- 8bit register.

- sfr address is 99h.

- Only byte-wise accessable.

- The data is transmitted or received through SBUF in UART peripheral.

- Stack pointer(SP):

- 8bit register.

- sfr address is 81h.

- default value is 07h.

- only byte –wise accessable.

- SP holds the address of a stack.

- Timer/counter mode control register(TMOD):

- 8bit register.

- sfr address is 89h.

- default value is 00h.

- only byte-wise accessable.

- used to select the timer /counter function and mode of operation.

- Timer/counter control register(TCON):

- 8bit register.

- sfr address is 88h.

- default value is 00h.

- both bit and byte-wise accessable.

- used to control the timer/counter and read the status of timer/counter flags.

16bit

| 00H | 00H |
| --- | --- |

T0
TH0 (8CH)                                        TL0(8AH)

8bit	8bit

16bit

| 00H | 00H |
| --- | --- |

T1
     TH1 (8DH)                                            TL1(8BH)

  - TL0,TH0,TL1,TH1 registers are only byte-wise accessable.

  - The four registers are called as count registers of timers/counters.

Q1)    How many sfr’s are accessed both bit and byte-wise?

ans) 11sfr’s(A,B,IE,IP,P0,P1,P2,P3,PSW,TCON,SCON)

Q2)  How many sfr’s are accessed only byte-wise?

ans) 10sfr’s (DPL,DPH,PCON,SP,TMOD,SBUF,TL0,TH0,TL1,TH1)